##### Top 20 Exam Questions

- Identify the chip responsible for USB communication on the Uno.

- Why does the ATmega328P not require an external programmer? (Ans: Bootloader).

- Calculate the voltage if analogRead() returns 512. (Ans: 2.5V).

- What is the purpose of the 16MHz Quartz Crystal?

- Which pin symbol indicates PWM capability?

- Explain "Active-Low" relay logic.

- What happens if the baud rate in Serial.begin() doesn't match the Serial Monitor?

- Identify the IC used as an Op-Amp in the HC-SR04.

- Name the sensor that uses a 40kHz ultrasonic burst.

- Define the spectral response of an LDR. (Ans: 400-700nm).

- How do you adjust the sensitivity of an IR sensor?

- What is the role of the Flyback Diode in a relay?

- Debugging: If an LED won't turn on, which IDE tool helps check logic values?

- How many digital pins are available on the Uno?

- What is the maximum voltage an Uno can withstand?

- Describe the difference between void setup() and void loop().

- Why are resistor networks used on the board?

- Which memory is used for runtime variables? (Ans: RAM).

- What does the "TX" LED signify?

- Define a "Polyfuse" and its benefit.